GMAW Pre-Test

| 1. What does GMAW stand for? |  |
| --- | --- |
| a. Gas Mig Arc Welding | b. Gas Mixture Arc Welding |
| c. Gas Metal Arc Welding | d. Gas Mechanical Arc Welding |
| 2. GMAW is generally done with what polarity? |  |
| a. Direct Current Electrode Negative (DC-) | b. Direct Current Electrode Positive (DC+) |
| c. Alternating Current (AC) | d. Does not matter |
| 3. What type of power source is generally used for GMAW welding? |  |
| a. Constant Current | b. Variable Voltage |
| c. Alternating Current | d. Constant Voltage |
| 4. What does the "S" stand for in the GMAW classification for ER70S-6? |  |
| a. Solid | b. Strength |
| c. Silicon | d. Short Circuit |
| 5. What does the "70" stand for in the GMAW classification for ER70S-3? |  |
| a. Trade name | b. Yield Strength |
| c. Tensile Strength | d. Hardness |
| 6. What mode of metal transfer allows for consistent all position GMAW welding? |  |
| a. Axial Spray | b. Short Circuit |
| c. Spray Arc | d. Globular |
| 7. During short circuit transfer, what does increasing the inductance do? |  |
| a. Slows down the rate of current rise when the short is produced. |  |
| b. Consumes less input power thus making the machine more energy efficient. |  |
| c. Increase the rate of current rise when the short is produced. |  |
| d. Improves arc starting. |  |
| 8. During spray transfer, what does increasing the inductance do? |  |
| a. Slows down the rate of current rise when the short is produced. |  |
| b. Consumes less input power thus making the machine more energy efficient. |  |
| c. Increase the rate of current rise when the short is produced. |  |
| d. Nothing. |  |
| 9. What gas mixture is generally associated with short circuit transfer on steel? |  |
| a. 100% CO2 | b. 100% AR |
| c. 75% AR/25% CO2 | d. 75% CO2/25% AR |
| 10. What gas mixture is generally associated with globular transfer on steel? |  |
| a. 100% CO2 | b. 100% AR |
| c. 75% AR/25% CO2 | d. 90% AR/10% CO2 |
| 11. What gas mixture is generally associated with spray transfer on steel? |  |
| a. 100% CO2 | b. 100% AR |
| c. 75% AR/25% CO2 | d. 90% AR/10% CO2 |
| 12. Inadequate gas flow can cause what defect in GMAW welding? |  |
| a. Undercut | b. Porosity |
| c. Worm tracking | d. Hydrogen cracking |
| 13. What is generally not an effect of a worn or loose contact tip? |  |
| a. Poor arc starts | b. Excessive spatter |
| c. Ropey welds | d. Porosity |
| 14. What is a benefit of copper coated GMAW wires? |  |
| a. Better arc starts | b. Improved Feedability |
| c. Increased contact tip life | d. All of the above |
| 15. The industry rating on air-cooled GMAW guns is 60% with what gas? |  |
| a. 100% CO2 | b. 100% AR |
| c. 75% AR/25% CO2 | d. 90% AR/10% CO2 |
| 16. The industry rating on air-cooled GMAW guns as argon content increases? |  |
| a. Decreases | b. Increases |
| c. Remains the same | d. Shielding gas has no effect on rating |
| 17. As a rule of thumb, what is the limitation of steel thickness for short circuit transfer? |  |
| a. 3/4" | b. 3/16" |
| c. 14 gauge | d. 1" |
| 18. What does STT stand for? |  |
| a. Short Circuit Tension Transfer | b. Surface Tension Transfer |
| c. Solid Tension Transfer | d. Short Tension Transfer |
| 19. What is not an advantage to pulse welding? |  |
| a. All position welding | b. Reduced spatter |
| c. Reduced fume emission | d. All are advantages |
| 20. As a general rule, what is the minimum argon content for pulse welding? |  |
| a. 80% | b. 85% |
| c. 90% | d. 95% |
| 21. Silicon and manganese are added to GMAW wires for what purpose? |  |
| a. Rust inhibitors | b. Better out of position welding |
| c. Deoxidizers | d. No effect on welding performance |
| 22. Which GMAW wire would be the best choice for welding scaled or rusty carbon steel? |  |
| a. ER70S-6 | b. ER70S-3 |
| c. ER70S-4 | d. ER70S-G |
| 23. What type of arc transfer is associated with STT? |  |
| a. Pulse | b. Spray |
| c. Short Circuit | d. Globular |
| 24. Cold lap or lack of fusion is generally associated with what type of arc transfer? |  |
| a. Short Circuit | b. STT |
| c. Globular | d. Spray |
| 25. Arc welding problems can generally be traced back to what possible cause? |  |
| a. Poor electrical connections | b. Worn contact tips |
| c. Dirty/clogged liners | d. All of the above |