GTAW Post-Test

| 1. What is a proper gas flow rate for use in GTAW application? |  |
| --- | --- |
| a. 50 cfh | b. 20 cfh |
| c.20 cfm | d. 50 psi |
| 2. The AWS Classification of GTAW filler metal, ER70S-6, what does the "R" represent? |  |
| a. Round | b. Radium |
| c. Rod | d. Rectified |
| 3. What is one of the primary causes of porosity in GTAW? |  |
| a. Hydrogen Pick-up | b. Oxygen Pick-up |
| c. Nitrogen Pick-up | d. High Current Level |
| 4. GTAW of magnesium is done using what polarity? |  |
| a. DC+ | b. AC |
| c. DC- | d. All of the Above |
| 5. What is not an advantage of argon vs. helium as a shielding gas in GTAW? |  |
| a. Argon = Smoother and Quieter Arc | b. Argon = Greater Cleaning Action on AC |
| c. Argon = Smaller Heat-Affected Zone | d. Argon = Easier Arc Starts |
| 6. What is the proper electrical stickout of tungsten in most GTAW applications? |  |
| a. 1" | b. 3/4" |
| c. 1/2" | d. 1/8" |
| 7. What is not true regarding GTAW using alternating current? |  |
| a. DC+ Cleans Oxides from Base Material | b. High Frequency is for Arc Starts Only |
| c. DC- Penetrates Base Material | d. High Frequency is Continuos while Welding |
| 8. What is the proper type of power source for GTAW? |  |
| a. Constant Coltage | b. Constant Power |
| c. Constant Current | d. None of the Above |
| 9. What polarity has electrons flowing from the tungsten to the work piece? |  |
| a. DC- | b. DC+ |
| c. AC | d. None of the Above |

| 10. When using GTAW, what is not a recommended practice for adding filler metal? |  |
| --- | --- |
| a. Add at 15o Angle to Work Piece | b. Add to Front Edge of Weld Puddle |
| c. Remove Filler from Weld Zone to Cool | d. All of the Above are Recommended |
| 11. What is not an advantage to GTAW pulse welding? |  |
| a. Deeper Penetration | b. Less Distortion |
| c. Lower Heat-Input | d. Ability to Weld Gaps/Open Root |
| 12. In GTAW, what is the proper tungsten diameter for 1/4" steel? |  |
| a. 1/16" | b. 1/8" |
| c. 3/32" | d. 1/2" |
| 13. What determines the voltage when using GTAW? |  |
| a. Tungsten Electrical Stick Out | b. Arc Length |
| c. Shielding Gases | d. Travel Speed |
| 14. What is not an advantage to an inverter machine versus a conventional machnine? |  |
| a. Lower Input Current Draw | b. Higher Energy Efficiency |
| c. Smaller Size | d. All are Advantages |
| 15. What is not a recommended procedure for reducing high frequency emmission? |  |
| a. Use Natural Rubber Cables | b. Disconnect Unnecessary Leads |
| c. Use a Water Cooled Torch | d. Keep Cables in Good Repair |
| 16. Which of the following gases is not an inert gas? |  |
| a. Helium | b. Carbon Dioxide |
| c. Argon | d. All of the Above |
| 17. GTAW of chrome-moly is done using what polarity? |  |
| a. DC- | b. AC |
| c. Argon | d. None of the Above |
| 18. Why is DC+ polarity not commonly used in GTAW? |  |
| a. 70% of Heat Focussed on Work Piece | b. 30% of Heat Focussed on Tungsten |
| c. Overheating of Tungsten Can Occur | d. Penetration to Deep for Thin Materials |
| 19. What is a cause of tungsten "frosting" or discoloration? |  |
| a. Inadequate Shielding Gas | b. Excessive Arc Length |
| c. Excessive Current Levels | d. Wrong Tungsten for Application |
| 20. Water-cooled GTAW torches generally have how many cables? |  |
| a. 1 | b. 2 |
| c. 3 | d. 4 |
| 21. What is not a function of inert shielding gas for GTAW of Aluminum? |  |
| a. Provide Controlled Ionization Path | b. Protect Weld from Contamination |
| c. Cleans the Oxides Off Stainless Steels | d. All of the Above are Functions |
| 22. What is not true when using 4043 versus 5356 in GTAW of aluminum? |  |
| a. 4043 Produces a Shinier Weld Bead | b. 4043 is Alloyed with Magnesium |
| c. 5356 is Alloyed with Magnesium | d. 5356 is Stronger |
| 23. When changing from Argon to Argon/Helium, what procedure needs adjustment? |  |
| a. Lower Current Level | b. Change Polarity |
| c. Increase the Current Level | d. None of the Above |
| 24. When using GTAW, what is not an andvatage? |  |
| a. No Slag | b. X-ray quality welds |
| c. All Positiion Welding | d. Excellent Outdoor Welding Capability |
| 25. What is not recommended for preparing aluminum prior to welding? |  |
| a. Clean with a Degreasing Solvent | b. Wire Brush with a Carbon Steel Brush |
| c. Wire Brush with a Stainless Steel Brush | d. Scrape Aluminum to Remove Oxide Coating |