GMAW

GMAW – Gas Metal Arc Welding

- Uses argon gas to “shield” the weld from other gasses in the atmosphere

- Voltage is set on the machine

- Amperage is adjusted by changing wire feed speed

# Weave Pattern

Weave can control:

- Penetration

- Buildup

- Width

- Porosity

- Undercut

- Overlap

- Slag inclusions

The weave is usually the personal choice of the welder

Reminder:  with a circular or oval weaving pattern, the electrode is in the middle twice as often as it is at both sides – adjust speed to accommodate this reality