Shielded Metal Arc Welding

SMAW – Shielded Metal Arc Welding

- Current flows from neg to pos

- Resistance to current causes heat

- Air gap btwn electrode and work causes much resistance

- Temp. 11,000 degrees F

- Electrodes have chemicals added to coverings to stabilize arc

Currents

- DCEN – Direct Current Electrode Negative

- High Melting rate

- DCEP – Direct Current Electrode Positive

- Best welding arc characteristics

- AC – Alternating  Current

- Electrons change direction every 1/120  of a second

Duty Cycle – Percentage of time the welding machine can be used continuously -–60% = 6 of 10 min

# SMAW Settings

## Amperage –

## Too low

## = Poor fusion, poor arc stability, Little or no penetration of weld into base, Shorter arc length

- Too high

= Wide, flat, deep penetrated bead, Too much heat – chemicals burn out early

= Longer arc length

= Slag inclusions, Porosity

# Arc Length

Arc Length – the distance the arc must jump from the end of the electrode to the plate

Electrode must be lowered continuously to maintain constant arc length

Short arc length = bead that is narrow w/ high buildup

- Sticking also occurs

Long arc length = bead that is wide and has little buildup

- Produce more spatter

Factors affecting length – type of electrode, joint design, metal thickness, and current setting

# Striking the Arc

Scratch electrode across the plate  as arc is established slightly raise electrode to desired arc length  Once arc length is established, lower the electrode as it burns off

If the electrode sticks to the plate  quickly release it by squeezing the electrode holder

# Electrode Angle

## Work angle – the angle of the electrode relative to the work

- should always be at 90 degrees

- an off angle will push the bead to one side

Travel Angle – angle is measured from the electrode to the surface of the weld in the direction of travel

- Angle is used to heat metal before electrode gets there for better fusion

- Trailing angle – usually 20 – 30 degrees

- More common

- Lead angle – blows debris out of the way

# Welding Safety