Semester Test Review

Name:  __________________________

Correctable True and False

Directions:  Indicate whether the statement is true or false by circling the correct answer.  If the question is false, correct the part of the statement which makes it false.

1.  T     F   MIG welding is commonly referred to as a semiautomatic welding process.

2.  T     F   A common benefit to MIG welding is that welds can be done in all positions.

3.  T     F   A common disadvantage to MIG welding is that there is more post weld cleaning required.

4.  T     F   A gas used in Gas Metal Arc Welding is Nitrogen.

5.  T     F   The reason for the gas in GMAW is so there is a chemical reaction to cause heat.

6.  T     F   Voltage is adjusted by increasing or decreasing the wire feed speed.

7.    T     F  One thing that the covering (flux) on an electrode does is stabilizes the arc

8.   T     F  The resistance to the flow of electricity is what causes the heat that melts metal in SMAW.

9.     T     F   Welder burn of the eyes usually causes blindness.

10.   T     F   The oxyacetylene system can be used for cutting and welding.

Multiple Choice:  Choose the best answer for the following questions.

11.   Which way does electric current flow in SMAW?

- Positive to Negative

- Negative to Positive

- It changes constantly

- None of the above, SMAW does not have anything to do with electricity

12.The average duty cycle for a SMAW machine is…

- 60%

- 20%

- 90%

- 3%

13. However the duty cycle can be as low as…

- 60%

- 20%

- 90%

- 3%

14.  The work angle of a SMAW weld should be at what degree?

- 20

- 45

- 90

- It really doesn’t matter

15.The travel angle of a SMAW weld should be at about what degree?

- 45

- 30

- 90

- The travel angle doesn’t matter either

16.Welder burn is essentially like which of the following?

| a. An electric stimuli to the eye | b. A sun burn on the eye |
| --- | --- |
| c. A spark flying into the eye and scratching it | d. A heat burn caused by too much heat |

17.The kindling temperature of steel is:

| a. 900 degrees F | b. 212 degrees F |
| --- | --- |
| c. 1100 – 1600 degrees F | d. 1600 – 1800 degrees F |

18.During proper oxyacetylene cutting, how is the metal cut?

| a. It is melted apart | b. It is cut by forcing acetylene through it |
| --- | --- |
| c. It is cut by forcing oxygen through it | d. None of the above |

Short Answer/ Completion:  Answer each of the following questions in complete sentences.

19. What effect from the welder causes welder burn in the eyes?

20.Name 3 other sources of UV light causing eye irritation.

21. What is kindling temperature?

22. Which 2 gases does oxyacetylene cutting use?

23. Briefly explain the difference between soft and hard slag.

24. What are 2 characteristics of Direct Current Electrode Negative?

25. What are 2 characteristics of Direct Current Electrode Positive?

26. Name 2 characteristics of a weld done with the amperage set too low.

27. Name 2 characteristics of a weld done with the amperage set too high.

28.  What do you do if the electrode sticks to the plate while trying to strike the arc?

- What does ‘AWS’ stand for?

- What do the first two numbers of a 4-digit electrode number stand for?

Fill in the blank

- You have been given a piece of flat metal.  Now you have 10 minutes to go into the shop and weld a weave pattern using the arc welder and a weave pattern using the MIG welder.  Each of these welds must be at least 4 inches in length.  When you are finished welding, cool the piece of metal and put it on the bench by your name card.  You may practice on a piece of scrap metal prior to making these welds, but there should be only 2 welds on this particular piece of metal.   Each of these 2 welds will be worth 20 points.  I will grade the welds by the standards set forth throughout the semester.

-Arc weave pattern:  4” long, absence of air pockets, absence of slag inclusions, proper work & travel angles, proper/uniform weave spacing

-MIG weave pattern:  4” long, absence of air pockets, proper work & travel angles, proper/ uniform weave spacing,